Problem 1.11

Apply the conservation of volume (see Prob. 1.9) to simulate the level of liquid in a conical storage tank (Fig. P1.11). The liquid flows in at a sinusoidal rate of Qin = 3 sin2(t) and flows out according to

Qout = 3(y − yout)1.5                y > yout

Qout = 0                            y ≤ yout

where flow has units of m3/d and y = the elevation of the water surface above the bottom of the tank (m). Use Euler’s method to solve for the depth y from t = 0 to 10 d with a step size of 0.5 d. The parameter values are rtop = 2.5 m, ytop = 4 m, and yout = 1 m. Assume that the level is initially below the outlet pipe with y(0) = 0.8 m.

Problem 1.9

A storage tank (Fig. P1.9) contains a liquid at depth y, where y = 0 when the tank is half full. Liquid is withdrawn at a constant flow rate Q to meet demands. The contents are resupplied at a sinusoidal rate 3Q sin2(t).

Problem continued on next page...

Equation (1.14) can be written for this system as

or, since the surface area A is constant

Use Euler’s method to solve for the depth y from t = 0 to 10 d with a step size of 0.5 d. The parameter values are A = 1250 m2 and Q = 450 m3/d. Assume that the initial condition is y = 0.