Problem 1.21

As noted in Sec. 1.4, a fundamental representation of the drag force, which assumes turbulent conditions (i.e., a high Reynolds number), can be formulated as

where Fd = the drag force (N), ρ = fluid density (kg/m3), A = the frontal area of the object on a plane perpendicular to the direction of motion (m2), υ = velocity (m/s), and Cd = a dimensionless drag coefficient.

(a) Write the pair of differential equations for velocity and position (see Prob. 1.19) to describe the vertical motion of a sphere with diameter, d (m), and a density of ρs (kg/m3). The differential equation for velocity should be written as a function of the sphere’s diameter.

(b) Use Euler’s method with a step size of Δt = 2 s to compute the position and velocity of a sphere over the first 14 seconds. Employ the following parameters in your calculation: d = 120 cm, ρ = 1.3 kg/m3, ρs = 2700 kg/m3, and Cd = 0.47. Assume that the sphere has the initial conditions: x(0) = 100 m and υ(0) = −40 m/s.

(c) Develop a plot of your results (i.e., y and υ versus t) and use it to graphically estimate when the sphere would hit the ground.

(d) Compute the value for the bulk second-order drag coefficient,  (kg/m). Note that the bulk second-order drag coefficient is the term in the final differential equation for velocity that multiplies the term υ |υ|.

Problem 1.19

The velocity is equal to the rate of change of distance, x (m):

Eq. P1.19:

Use Euler’s method to numerically integrate Eqs. (P1.19) and (1.8) in order to determine both the velocity and distance fallen as a function of time for the first 10 seconds of freefall using the same parameters and conditions as in Example 1.2. Develop a plot of your results.

Eq. 1.8: