Problem 3.24

Develop a MATLAB script to compute the velocity, υ, and position, z, of a hot air balloon as described in Prob. 1.28. Perform the calculation from t = 0 to 60 s with a step size of 1.6 s. At z = 200 m, assume that part of the payload (100 kg) is dropped out of the balloon. Your script should be structured like:

% YourFullName

% Hot Air Balloon Script

clear,clc,clf

g = 9.81;

global g

% set parameters

r = 8.65; % balloon radius

CD = 0.47; % dimensionless drag coefficient

mP = 265; % mass of payload

P = 101300; % atmospheric pressure

Rgas = 287; % Universal gas constant for dry air

TC = 100; % air temperature

rhoa = 1.2; % air density

zd = 200; % elevation at which mass is jettisoned

md = 100; % mass jettisoned

ti = 0; % initial time (s)

tf = 60; % final time (s)

vi = 0; % initial velocity

zi = 0; % initial elevation

dt = 1.6; % integration time step

% precomputations

d = 2 * r; Ta = TC + 273.15; Ab = pi / 4 * d ^ 2;

Vb = pi / 6 * d ^ 3; rhog = P / Rgas / Ta; mG = Vb * rhog;

FB = Vb * rhoa * g; FG = mG * g; cdp = rhoa * Ab * CD / 2;

% compute times, velocities and elevations

[t,y] = Balloon(FB, FG, mG, cdp, mP, md, zd, ti,vi,zi,tf,dt);

% Display results

Your code to display a nice labeled table of times, velocities, and elevations

% Plot results

Your code to create a nice labeled plot of velocity and elevation versus time.

Problem continued on next page...

Your function should be structured like:

function [tout,yout]=Balloon(FB,FG,mG,cdp,mP,md,zd,ti,vi,zi,tf,dt)

global g

% balloon

% function [tout,yout]=Balloon(FB, FG, mG, cdp, mP1, md, zd, ti,vi,zi,tf,dt)

% Function to generate solutions of vertical velocity and elevation

% versus time with Euler’s method for a hot air balloon

% Input:

% FB = buoyancy force (N)

% FG = gravity force (N)

% mG = mass (kg)

% cdp=dimensional drag coefficient

% mP= mass of payload (kg)

% md=mass jettisoned (kg)

% zd=elevation at which mass is jettisoned (m)

% ti = initial time (s)

% vi=initial velocity (m/s)

% zi=initial elevation (m)

% tf = final time (s)

% dt=integration time step (s)

% Output:

% tout = vector of times (s)

% yout[:,1] = velocities (m/s)

% yout[:,2] = elevations (m)

% Code to implement Euler’s method to compute output and plot results

Problem continued on next page...

Problem 1.28

Figure P1.28 shows the forces exerted on a hot air balloon system.

Formulate the drag force as

where ρa = air density (kg/m3), υ = velocity (m/s), A = projected frontal area (m2), and Cd = the dimensionless drag coefficient (≅ 0.47 for a sphere). Note also that the total mass of the balloon consists of two components:

m = mG + mP

where mG = the mass of the gas inside the expanded balloon (kg), and mP = the mass of the payload (basket, passengers, and the unexpanded balloon = 265 kg). Assume that the ideal gas law holds (P = ρRT), that the balloon is a perfect sphere with a diameter of 17.3 m, and that the heated air inside the envelope is at roughly the same pressure as the outside air.

Other necessary parameters are:

Normal atmospheric pressure, P = 101,300 Pa

The gas constant for dry air, R = 287 Joules/(kg K)

Problem continued on next page...

The air inside the balloon is heated to an average temperature, T = 100 ºC

The normal (ambient) air density, ρ = 1.2 kg/m3.

(a) Use a force balance to develop the differential equation for dυ/dt as a function of the model’s fundamental parameters.

(b) At steady-state, calculate the particle’s terminal velocity.

(c) Use Euler’s method and Excel to compute the velocity from t = 0 to 60 s with Δt = 2 s given the previous parameters along with the initial condition: υ(0) = 0. Develop a plot of your results.