Problem 3.28

The pressure and temperature of the atmosphere are constantly changing depending on a number of factors including altitude, latitude/longitude, time of day, and season. To take all these variations into account when considering the design and performance of flight vehicles is impractical. Therefore, a standard atmosphere is frequently used to provide engineers and scientists with a common reference for their research and development. The International Standard Atmosphere is one such model of how conditions of the earth’s atmosphere change over a wide range of altitudes or elevations. The following table shows values of temperature and pressure at selected altitudes.

The temperature at each altitude can then be computed as

T(h) = Ti + γi (h − hi )                                        hi < h ≤ hi+1

where T(h) = temperature at altitude h (°C), Ti = the base temperature for layer i (°C), γi = lapse rate or the rate at which atmospheric temperature decreases linearly with increase in altitude for layer i (°C/km), and hi = base geopotential altitude above mean sea level (MSL) for layer i. The pressure at each altitude can then be computed as

Problem continued on next page...

where p(h) = pressure at altitude h(Pa ≡ N/m2), pi = the base pressure for layer i (Pa). The density, ρ(kg/m3), can then be calculated according to a molar form of the ideal gas law:

where M = molar mass (≅ 0.0289644 kg/mol), R = the universal gas constant (8.3144621 J/(mol · K), and Ta = absolute temperature (K) = T + 273.15.

Develop a MATLAB function, StdAtm, to determine values of the three properties for a given altitude. If the user requests a value outside the range of altitudes, have the function display an error message and terminate the application. Use the following script as the starting point to create a 3-panel plot of altitude versus the properties.

% Script to generate a plot of temperature, pressure and density

% for the International Standard Atmosphere

clc,clf

h = [0 11 20 32 47 51 71 84.852];

gamma = [−6.5 0 1 2.8 0 −2.8 −2];

T = [15 −56.5 −56.5 -44.5 −2.5 −2.5 −58.5 −86.28];

p = [101325 22632 5474.9 868.02 110.91 66.939 3.9564 0.3734];

hint = [0:0.1:84.852];

for i = 1:length(hint)

[Tint(i),pint(i),rint(i)] = StdAtm(h,T,p,gamma,hint(i));

end

% Create plot

% Function call to test error trap

[Tint(i),pint(i),rint(i)]=StdAtm(h,T,p,gamma,85);