Problem 3.29

Develop a MATLAB function to convert a vector of temperatures from Celsius to Fahrenheit and vice versa. Test it with the following data for the average monthly temperatures at Death Volley, CA and at the South Pole.

| Day | Death Valley°F | South Pole °C |
| --- | --- | --- |
| 15 | 54 | −27 |
| 45 | 60 | −40 |
| 75 | 69 | −53 |
| 105 | 77 | −56 |
| 135 | 87 | −57 |
| 165 | 96 | −57 |
| 195 | 102 | −59 |
| 225 | 101 | −59 |
| 255 | 92 | −59 |
| 285 | 78 | −50 |
| 315 | 63 | −38 |
| 345 | 52 | −27 |

Use the following script as the starting point to generate a 2-panel stacked plot of the temperatures versus day for both of the sites with the Celsius time series at the top and the Fahrenheit at the bottom. If the user requests a unit other than 'C' or 'F', have the function display an error message and terminate the application.

Problem continued on next page...

% Script to generate stacked plots of temperatures versus time

% for Death Valley and the South Pole with Celsius time series

% on the top plot and Fahrenheit on the bottom.

clc,clf

t=[15 45 75 105 135 165 195 225 255 285 315 345];

TFDV = [54 60 69 77 87 96 102 101 92 78 63 52];

TCSP = [−27 −40 −53 −56 −57 −57 −59 −59 −59 −50 −38 −27];

TCDV=TempConv(TFDV,'C');

TFSP=TempConv(TCSP,'F');

% Create plot

% Test of error trap

TKSP=TempConv(TCSP,'K');