Solution 3.24

% YourFullName

% Hot Air Balloon Script

clear,clc,clf

global g

% set parameters

g=9.81;

r=8.65; % balloon radius

CD=0.47; % dimensionless drag coefficient

mP=265; % mass of payload

P=101300; % atmospheric pressure

Rgas=287; % Universal gas constant for dry air

TC=100; % air temperature

rhoa=1.2; % air density

zd=200; % elevation at which mass is jettisoned

md=100; % mass jettisoned

ti=0; % initial time (s)

tf=60; % final time (s)

vi=0; % initial velocity

zi=0; % initial elevation

dt=1.6; % integration time step

% precomputations

d = 2 * r; Ta = TC + 273.15; Ab = pi / 4 * d ^ 2;

Vb = pi / 6 * d ^ 3; rhog = P / Rgas / Ta; mG = Vb * rhog;

FB = Vb * rhoa * g; FG = mG * g; cdp = rhoa * Ab * CD / 2;

% compute times, velocities and elevations

[t,y] = Balloon(ti,vi,zi,tf,dt,FB,FG,mG,cdp,mP,md,zd);

% Display results

fprintf('      t(s)     v(m/s)     z(m/s)\n');

xx=[t y];

fprintf('%10.3f %10.3f %10.3f\n',xx');

% Plot results

subplot(2,1,1)

plot(t,y(:,1))

title('Velocity vs. time')

xlabel('time (s)'),ylabel('velocity (m/s)')

subplot(2,1,2)

plot(t,y(:,2))

title('Elevation vs. time')

xlabel('time (s)'),ylabel('elevation (m)')

Solution continued on next page...

function [tout,yout]=Balloon(ti,vi,zi,tf,dt,varargin)

% balloon

% function [tout,yout]=Balloon(ti,vi,zi,tf,dt,FB,FG,mG,cdp,mP,md,zd)

% Function to generate solutions of vertical velocity and elevation

% versus time with Euler’s method for a hot air balloon

% Input:

% FB = buoyancy force (N)

% FG = gravity force (N)

% mG = mass (kg)

% cdp=dimensional drag coefficient

% mP= mass of payload (kg)

% md=mass jettisoned (kg)

% zd=elevation at which mass is jettisoned (m)

% ti = initial time (s)

% vi=initial velocity (m/s)

% zi=initial elevation (m)

% tf = final time (s)

% dt=integration time step (s)

% Output:

% tout = vector of times (s)

% yout[:,1] = velocities (m/s)

% yout[:,2] = elevations (m)

% Code to implement Euler’s method to compute output

t = ti; y = [vi zi];

tout = t; yout = y;

while(1)

if t + dt > tf, dt = tf - t; end

dy=dydt(y,varargin{:});

y = y + dy' * dt;

t = t + dt;

tout = [tout; t]; yout = [yout; y];

if t >= tf, break, end

end

end

function [dy]=dydt(y,FB,FG,mG,cdp,mP,md,zd)

global g

if y(2) < zd

mP1 = mP;

else

mP1 = mP - md;

end

dy = [(FB - FG) / (mP1 + mG) - mP1 * g / (mP1 + mG) - ...

cdp / (mP1 + mG) * y(1) ^ 2; y(1)];

end

Solution continued on next page...

Output:

t(s)     v(m/s)     z(m/s)

0.000      0.000      0.000

1.600      2.352      0.000

3.200      4.496      3.762

4.800      6.090     10.956

6.400      7.051     20.699

8.000      7.539     31.981

9.600      7.760     44.043

11.200      7.854     56.459

12.800      7.893     69.025

14.400      7.909     81.655

16.000      7.916     94.309

17.600      7.918    106.975

19.200      7.920    119.644

20.800      7.920    132.316

22.400      7.920    144.988

24.000      7.920    157.660

25.600      7.920    170.332

27.200      7.920    183.005

28.800      7.920    195.677

30.400      7.920    208.350

32.000      8.495    221.022

33.600      8.704    234.615

35.200      8.773    248.541

36.800      8.795    262.577

38.400      8.802    276.649

40.000      8.804    290.732

41.600      8.805    304.818

43.200      8.805    318.906

44.800      8.805    332.994

46.400      8.805    347.082

48.000      8.805    361.171

49.600      8.805    375.259

51.200      8.805    389.347

52.800      8.805    403.435

54.400      8.805    417.523

56.000      8.805    431.612

57.600      8.805    445.700

59.200      8.805    459.788

60.000      8.805    466.832

Solution continued on next page...