Problem 5.3

Figure P5.3 shows a pinned-fixed beam subject to a uniform load. The equation for the resulting deflections is

Develop a MATLAB script that

(a) plots the function, dy/dx versus x (with appropriate labels) and

(b) uses LastNameBisect to determine the point of maximum deflection (i.e., the value of x where dy/dx = 0). Then substitute this value into the deflection equation to determine the value of the maximum deflection. Employ initial guesses of xl = 0 and xu = 0.9L. Use the following parameter values in your computation (making sure that you use consistent units): L = 400 cm, 
E = 52,000 kN/cm2, I = 32,000 cm4, and w = 4 kN/cm. In addition, use Ead = 0.0000001 m. Also, set format long in your script so you display 15 significant digits for your results.