Solution 5.3

The derivative of the function is

Therefore, to find the maximum deflection, we need to find the location where the derivative is zero. That is, the point of maximum deflection is the root of

The following script can be developed to determine the root with your bisection function,

clear;clc;clf

w=4*1e3*100;L=400/100;E=52000*1e3*1e4;I=32000/100^4;

f = @(x,w,E,I,L) -w/(48*E*I)*(8*x.^3-9*L*x.^2+L^3);

d = @(xr,w,E,I,L) -w/(48*E*I)*(2*xr.^4-3*L*xr.^3+L^3*xr);

subplot(2,1,1)

xp=[0:L/16:L];

dydxp=f(xp,w,E,I,L);

plot(xp,dydxp),grid

xlabel('x(m)'),ylabel('dy/dx')

title('Slope versus distance')

subplot(2,1,2)

yp=d(xp,w,E,I,L);

plot(xp,yp),grid

xlabel('x(m)'),ylabel('y(m)')

title('Deflection versus distance')

format long, format compact

[root,Ea,ea,n] =bisectnew(f,0,0.9*L,0.0000001,w,E,I,L)

max_deflection=d(root,w,E,I,L)

Solution continued on next page...

Here is the bisection function

function [root,Ea,ea,n] = bisectnew(func,xl,xu,Ead,varargin)

% bisection roots zero

% [root,Ea,ea,n] = bisectnew(func,xl,xu,Ead,varargin)

%   uses bisection method to find the root of a function

%   with a fixed number of iterations to attain

%   a prespecified tolerance

% input:

%   func = name of function

%   xl, xu = lower and upper guesses

%   Ead = (optional) desired tolerance (default = 0.000001)

%   p1,p2,... = additional parameters used by func

% output:

%   root = real root

%   Ea = absolute error

%   ea = % relative error

%   n = iterations

if func(xl,varargin{:})*func(xu,varargin{:})>0

%if guesses do not bracket a sign change

disp('no bracket')    %display an error message

return                %and terminate

end

% if necessary, assign default values

if nargin<4|isempty(Ead),Ead=0.000001;end %if Ead blank set to 0.000001

xr = xl;

% compute n and round up to next highest integer

n = ceil(log2((xu - xl)/Ead));

for i = 1:n

xrold = xr;

xr = (xl + xu)/2;

if xr ~= 0, ea = abs((xr - xrold)/xr) * 100; end

Ea = abs(xr - xrold);

test = func(xl,varargin{:})*func(xr,varargin{:});

if test < 0

xu = xr;

elseif test > 0

xl = xr;

else

ea = 0;

end

end

root = xr;

When the script is run, the result is

root =

1.686140710115433

Ea =

5.364418020903372e-08

ea =

3.181477078823466e-06

n =

26

max_deflection =

-0.003332997911279

Solution continued on next page...