Solution 5.14

The function to be solved is

(a) A plot of the function indicates a root at about x = 16.

(b) The shape of the function indicates that false position would be a poor choice (recall Fig. 5.9). Bisection with initial guesses of 0 and 20 can be used to determine a root of 15.85938 after 8 iterations with a = 0.493%. Note that false position would have required 68 iterations to attain comparable accuracy.

| i | xl | xu | xr | f(xl) | f(xr) | f(xl)f(xr) | a |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 20 | 10 | -0.01592 | -0.01439 | 0.000229 | 100.000% |
| 2 | 10 | 20 | 15 | -0.01439 | -0.00585 | 8.4210-5 | 33.333% |
| 3 | 15 | 20 | 17.5 | -0.00585 | 0.025788 | -0.00015 | 14.286% |
| 4 | 15 | 17.5 | 16.25 | -0.00585 | 0.003096 | -1.810-5 | 7.692% |
| 5 | 15 | 16.25 | 15.625 | -0.00585 | -0.00228 | 1.3310-5 | 4.000% |
| 6 | 15.625 | 16.25 | 15.9375 | -0.00228 | 0.000123 | -2.810-7 | 1.961% |
| 7 | 15.625 | 15.9375 | 15.78125 | -0.00228 | -0.00114 | 2.5910-6 | 0.990% |
| 8 | 15.78125 | 15.9375 | 15.85938 | -0.00114 | -0.00052 | 5.9810-7 | 0.493% |