Solution 5.15

This problem can be solved by determining the root of the derivative of the elastic curve

Therefore, after substituting the parameter values (in meter-kilogram-second units), we must determine the root of

f(x) = –5x4 + 216x2 – 1296 = 0

A plot of the function indicates a root at about x = 2.6 m.

Using initial guesses of 0 and 5 m, bisection can be used to determine the root. Here are the first few iterations:

| i | xl | xu | xr | f(xl) | f(xr) | f(xl)f(xr) | a |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 5 | 2.5 | -1296.00 | -141.31 | 183141 |  |
| 2 | 2.5 | 5 | 3.75 | -141.31 | 752.73 | -106370 | 33.33% |
| 3 | 2.5 | 3.75 | 3.125 | -141.31 | 336.54 | -47557 | 20.00% |
| 4 | 2.5 | 3.125 | 2.8125 | -141.31 | 99.74 | -14095 | 11.11% |
| 5 | 2.5 | 2.8125 | 2.65625 | -141.31 | -20.89 | 2952 | 5.88% |

After 22 iterations, the root is determined as x = 2.683282852172852 m. This value can be substituted into Eq. (P5.15) to compute the maximum deflection as –0.005151900620158 m.