Solution 5.20

The solution can be formulated as

We want our program to work for Reynolds numbers between 2,500 and 1,000,000. Therefore, we must determine the friction factors corresponding to these limits. This can be done with any root location method to yield 0.011525 and 0.002913. Therefore, we can set our initial guesses as xl = 0.0028 and xu = 0.012. Equation (5.6) can be used to determine the number of bisection iterations required to attain an absolute error less than 0.000005,

which can be rounded up to 11 iterations. Here is a MATLAB function that is set up to implement 11 iterations of bisection to solve the problem.

function f=Fanning(func,xl,xu,varargin)

test = func(xl,varargin{:})*func(xu,varargin{:});

if test>0,error('no sign change'),end

for i = 1:11

xr = (xl + xu)/2;

test = func(xl,varargin{:})*func(xr,varargin{:});

if test < 0

xu = xr;

elseif test > 0

xl = xr;

else

break

end

end

f=xr;

This can be implemented in MATLAB. For example,

>> vk=@(f,Re) 4*log10(Re*sqrt(f))-0.4-1/sqrt(f);

>> format long

>> f=Fanning(vk,0.0028,0.012,2500)

f =

0.01152832031250

Solution continued on next page...

Here are additional results for a number of values within the desired range. We have included the true value and the resulting error to verify that the results are within the desired error criterion of Ea < 510-6.

| Re | Root | Truth | Et |
| --- | --- | --- | --- |
| 2500 | 0.0115283203125 | 0.0115247638118 | 3.5610-6 |
| 3000 | 0.0108904296875 | 0.0108902285840 | 2.0110-7 |
| 10000 | 0.0077279296875 | 0.0077271274071 | 8.0210-7 |
| 30000 | 0.0058771484375 | 0.0058750482511 | 2.1010-6 |
| 100000 | 0.0045025390625 | 0.0045003757287 | 2.1610-6 |
| 300000 | 0.0036220703125 | 0.0036178949673 | 4.1810-6 |
| 1000000 | 0.0029123046875 | 0.0029128191460 | 5.1410-7 |