Problem 6.15

The Redlich-Kwong equation of state is given by

where R = the universal gas constant [= 0.518 kJ/(kg K)], T = absolute temperature (K), p = absolute pressure (kPa), and υ = the volume of a kg of gas (m3/kg). The parameters a and b are calculated by

where pc = 4600 kPa and Tc = 191 K. As a chemical engineer, you are asked to determine the amount of methane fuel that can be held in a 3-m3 tank at a temperature of−40 °C with a pressure of 65,000 kPa. Use a root-locating method of your choice to calculate υ and then determine the mass of methane contained in the tank.