Problem 6.35

A circular pipe made out of new cast iron is used to convey water at a volume flow rate of 
Q = 0.3 m3/s. Assume that the flow is steady and fully developed and the water is incompressible. The head loss, friction, and diameter are related by the Darcy-Weisbach equation,

where f = the friction factor (dimensionless), L = length (m), D = pipe inner diameter (m), V = velocity (m/s), and g = the gravitational constant (= 9.81 m/s2). The velocity can be related to flow by

Q = Ac V                      (P6.35.2)

where Ac = the pipe’s cross-sectional area (m2) = πD2/4 and the friction factor can be determined by the Colebrook equation. If you want the head loss to be less than 0.006 m per meter of pipe, develop a MATLAB function to determine the smallest diameter pipe to achieve this objective. Use the following parameter values: υ = 1.16 × 10–6 m2/s and ε = 0.4 mm.