Problem 6.37

As described in Sec. 1.4, for objects falling through fluids at very low speeds, the flow regime around the object will be laminar and the relationship between the drag force and velocity is linear. In addition, in such cases, the buoyancy force must also be included. For such cases, a force balance can be written as

(gravity)           (buoyancy)             (drag)

where υ = velocity (m/s), t = time (s), m = the mass of the particle (kg), g = the gravitational constant (= 9.81 m/s2), ρf = fluid density (kg/m3), V = particle volume (m3), and cd = the linear drag coefficient (kg/m). Note that the mass of the particle can be computed as Vρs, where ρs = the density of the particle (kg/m3). For a small sphere, Stokes developed the following formula for the drag coefficient, cd = 6πμr, where μ = the fluid’s dynamic viscosity (N s/m2), and r = the sphere’s radius (m).

You release an iron sphere at the surface of a container (x = 0) filled with honey (Fig. P6.37) and then measure how long it takes to settle to the bottom (x = L). Use this information to estimate the honey’s viscosity based on the following parameter values: ρf = 1420 kg/m3, ρs = 7850 kg/m3, r = 0.02 m, L = 0.5 m, and t(x = 0.5) = 3.6 s. Check the Reynolds number (Re = ρf υd/μ, where d = diameter) to confirm that laminar conditions occurred during the experiment.  [Hint: The problem can be solved by integrating Eq. (P6.37) two times to yield an equation for x as a function of t.]