Problem 6.38

As depicted in Fig. P6.38a, a scoreboard is suspended above a sports arena by two cables, pinned at A, B, and C. The cables are initially horizontal and of length L. After the scoreboard is hung, a free-body diagram at node B can be developed as shown in Fig. P6.38b. Assuming that the weight of each cable is negligible, determine the deflection, d (m), that results if the scoreboard weighs W = 9000 N. Also, compute how much each cable is elongated. Note that each cable obeys Hooke’s law such that the axial elongation is represented by L′ – L = FL /(AcE) where F = the axial force (N), Ac = the cable’s cross-sectional area (m2), and E = the modulus of elasticity (N/m2). Use the following parameters for your calculations: L = 45 m, Ac = 6.362 × 10–4 m2, and E = 1.5 × 1011 N/m2.