Problem 6.40

Modify the fzerosimp function (Fig. 6.10) so that it can be passed any function with a single unknown and uses varargin to pass the function’s parameters. Then test it with the following script to obtain a solution for pipe friction based on Case Study 6.3,

clc

format long, format compact

rho = 1.23;mu = 1.79e − 5;D=0.005;V = 40;e = 0.0015/1000;

Re = rho*V*D/mu;

g = @(f,e,D) 1/sqrt(f)+2*log10(e/(3.7*D)+2.51/(Re*sqrt(f)));

f = fzerosimp(@(x) g(x,e,D),0.008,0.08)