Solution 6.15

The coefficient, a and b, can be evaluated as

>> format long

>> R = 0.518;pc = 4600;Tc = 191;

>> a = 0.427*R^2*Tc^2.5/pc

a =

12.55778319740302

>> b = 0.0866*R*Tc/pc

b =

0.00186261539130

The solution, therefore, involves determining the root of

MATLAB can be used to generate a plot of the function and to solve for the root. One way to do this is to develop an M-file for the function,

function y = fvol(v)

R = 0.518;pc = 4600;Tc = 191;

a = 0.427*R^2*Tc^2.5/pc;

b = 0.0866*R*Tc/pc;

T = 273.15-40;p = 65000;

y = p - R*T./(v-b)+a./(v.*(v+b)*sqrt(T));

This function is saved as fvol.m. It can then be used to generate a plot

clear, clc, format compact

v = linspace(0.002,0.004);

fv = fvol(v);

plot(v,fv)

grid

Solution continued on next page...

Thus, a root is located at about 0.0028. The fzero function can be used to refine this estimate,

vroot = fzero('fvol',0.0028)

vroot =

0.002808408657034

The mass of methane contained in the tank can be computed as

mass = V/v = 3/0.002808409 = 1068.2206