Solution 6.26

This problem can be solved in a number of ways. One approach involves using the modified secant method. This approach is feasible because the Swamee-Jain equation provides a sufficiently good initial guess that the method is always convergent for the specified parameter bounds. The following functions implement the approach:

function ffact = prob0626(eD,ReN)

% prob0626: friction factor with Colebrook equation

%   ffact = prob0626(eD,ReN):

%      uses modified secant equation to determine the friction factor

%      with the Colebrook equation

% input:

%   eD = e/D

%   ReN = Reynolds number

% output:

%   ffact = friction factor

maxit=100;es=1e-8;delta=1e-5;

iter = 0;

% Swamee-Jain equation:

xr = 1.325 / (log(eD / 3.7 + 5.74 / ReN ^ 0.9)) ^ 2;

% modified secant method

while (1)

xrold = xr;

xr = xr - delta*xr*func(xr,eD,ReN)/(func(xr+delta*xr,eD,ReN)...

-func(xr,eD,ReN));

iter = iter + 1;

if xr ~= 0, ea = abs((xr - xrold)/xr) * 100; end

if ea <= es | iter >= maxit, break, end

end

ffact = xr;

function ff=func(f,eD,ReN)

ff = 1/sqrt(f) + 2*log10(eD/3.7 + 2.51/ReN/sqrt(f));

Here are implementations for the extremes of the parameter range:

>> prob0626(0.00001,4000)

ans =

0.0399

>> prob0626(0.05,4000)

ans =

0.0770

>> prob0626(0.00001,1e7)

ans =

0.0090

>> prob0626(0.05,1e7)

ans =

0.0716