Problem 7.31

The Streeter-Phelps model can be used to compute the dissolved oxygen concentration in a river below a point discharge of sewage (Fig. P7.31),

(P7.31)

where o = dissolved oxygen concentration (mg/L), os = oxygen saturation concentration (mg/L), t = travel time (d), Lo = biochemical oxygen demand (BOD) concentration at the mixing point (mg/L), kd = rate of decomposition of BOD (d−1), ks = rate of settling of BOD (d−1), ka = reaeration rate (d−1), and Sb = sediment oxygen demand (mg/L/d).

As indicated in Fig. P7.31, Eq. (P7.31) produces an oxygen “sag” that reaches a critical minimum level oc, some travel time tc below the point discharge. This point is called “critical” because it represents the location where biota that depend on oxygen (like fish) would be the most stressed.

Develop a MATLAB script that

(a) generates a plots of the function versus travel time and

(b) uses fminbnd to determine the critical travel time and concentration,

given the following values:

os = 10 mg/L

kd = 0.1 d−1

ka = 0.6 d−1

ks = 0.05 d−1

Lo = 50 mg/L

Sb = 1 mg/L/d