Problem 7.44

As depicted in Fig. P7.44, a mobile fire hose projects a stream of water onto the roof of a building. At what angle, θ, and how far from the building, x1, should the hose be placed in order to maximize the coverage of the roof; that is, to maximize: x2 – x1? Note that the water velocity leaving the nozzle has a constant value of 3 m/s regardless of the angle, and the other parameter values are h1 = 0.06 m, h2 = 0.2 m, and L = 0.12 m. [Hint: The coverage is maximized for the trajectory that just clears the top front corner. That is, we want to choose an x1 and θ that just clear the top corner while maximizing x2 – x1.]