Solution 7.7

(a) First, the golden ratio can be used to create the interior points,

The function can be evaluated at the interior points

Because f(x1) > f(x2), the maximum is in the interval defined by x2, x1 and xu where x1 is the optimum. The error at this point can be computed as

The process can be repeated and all the iterations summarized as

| i | xl | f(xl) | x2 | f(x2) | x1 | f(x1) | xu | f(xu) | d | xopt | a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | -2.0000 | -29.6000 | 0.2918 | 1.0416 | 1.7082 | 5.0075 | 4.0000 | -12.8000 | 3.7082 | 1.7082 | 134.16% |
| 2 | 0.2918 | 1.0416 | 1.7082 | 5.0075 | 2.5836 | 5.6474 | 4.0000 | -12.8000 | 2.2918 | 2.5836 | 54.82% |
| 3 | 1.7082 | 5.0075 | 2.5836 | 5.6474 | 3.1246 | 2.9361 | 4.0000 | -12.8000 | 1.4164 | 2.5836 | 33.88% |
| 4 | 1.7082 | 5.0075 | 2.2492 | 5.8672 | 2.5836 | 5.6474 | 3.1246 | 2.9361 | 0.8754 | 2.2492 | 24.05% |
| 5 | 1.7082 | 5.0075 | 2.0426 | 5.6648 | 2.2492 | 5.8672 | 2.5836 | 5.6474 | 0.5410 | 2.2492 | 14.87% |
| 6 | 2.0426 | 5.6648 | 2.2492 | 5.8672 | 2.3769 | 5.8770 | 2.5836 | 5.6474 | 0.3344 | 2.3769 | 8.69% |
| 7 | 2.2492 | 5.8672 | 2.3769 | 5.8770 | 2.4559 | 5.8287 | 2.5836 | 5.6474 | 0.2067 | 2.3769 | 5.37% |
| 8 | 2.2492 | 5.8672 | 2.3282 | 5.8853 | 2.3769 | 5.8770 | 2.4559 | 5.8287 | 0.1277 | 2.3282 | 3.39% |
| 9 | 2.2492 | 5.8672 | 2.2980 | 5.8828 | 2.3282 | 5.8853 | 2.3769 | 5.8770 | 0.0789 | 2.3282 | 2.10% |
| 10 | 2.2980 | 5.8828 | 2.3282 | 5.8853 | 2.3468 | 5.8840 | 2.3769 | 5.8770 | 0.0488 | 2.3282 | 1.30% |
| 11 | 2.2980 | 5.8828 | 2.3166 | 5.8850 | 2.3282 | 5.8853 | 2.3468 | 5.8840 | 0.0301 | 2.3282 | 0.80% |

The maximum value is f(2.3282) = 5.8853

Solution continued on next page...

(b) First, the function values at the initial values can be evaluated

f(x1) = f(1.75) = 5.1051

f(x2) = f(2) = 5.6

f(x3) = f(2.5) = 5.7813

and substituted into Eq. (7.10) to give x4 = 2.3341, which has a function value of f(2.3341) = 5.8852. Because the function value for the new point is higher than for the intermediate point (x2) and the new x value is to the right of the intermediate point, the lower guess (x1) is discarded. Therefore, for the next iteration,

f(x1) = f(2) = 5.6

f(x2) = f(2.3341) = 5.8852

f(x3) = f(2.5) = 5.7813

which can be substituted into Eq. (7.10) to give x4 = 2.3112, which has a function value of f(2.3112) = 5.8846. At this point, an approximate error can be computed as

The process can be repeated, with the results tabulated below:

| i | x1 | f(x1) | x2 | f(x2) | x3 | f(x3) | x4 | f(x4) | a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.7500 | 5.1051 | 2.0000 | 5.6000 | 2.5000 | 5.7813 | 2.3341 | 5.8852 |  |
| 2 | 2.0000 | 5.6000 | 2.3341 | 5.8852 | 2.5000 | 5.7813 | 2.3112 | 5.8846 | 0.99% |
| 3 | 2.0000 | 5.6000 | 2.3112 | 5.8846 | 2.3341 | 5.8852 | 2.3270 | 5.8853 | 0.68% |
| 4 | 2.3112 | 5.8846 | 2.3270 | 5.8853 | 2.3341 | 5.8852 | 2.3263 | 5.8853 | 0.03% |
| 5 | 2.3112 | 5.8846 | 2.3263 | 5.8853 | 2.3270 | 5.8853 | 2.3264 | 5.8853 | 0.00% |

Thus, after 5 iterations, the result is converging rapidly on the true value of f(x) = 5.8853 at x = 2.3264.