Solution 7.23

The velocity of a falling object with an initial velocity and first-order drag can be computed as

The vertical distance traveled can be determined by integration

where negative z is distance upwards. Assuming that z0 = 0, evaluating the integral yields

Applying the initial condition

Substituting and collecting terms gives

Solution continued on next page...

Therefore the solution to this problem amounts to determining the minimum of this function (since the most negative value of z corresponds to the maximum height). This function can be plotted using the given parameter values. As in the following graph (note that the ordinate values are plotted in reverse), the maximum height occurs after about 4.2 s and appears to be about 110 m.

Here is the result of using the golden-section search to determine the maximum height

| i | xl | f(xl) | x2 | f(x2) | x1 | f(x1) | xu | f(xu) | d | xopt | a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 1.9098 | -82.008 | 3.0902 | -105.171 | 5 | -108.922 | 3.0902 | 3.0902 | 61.80% |
| 2 | 1.9098 | -82.008 | 3.0902 | -105.171 | 3.8197 | -111.014 | 5 | -108.922 | 1.9098 | 3.8197 | 30.90% |
| 3 | 3.0902 | -105.171 | 3.8197 | -111.014 | 4.2705 | -111.790 | 5.0000 | -108.922 | 1.1803 | 4.2705 | 17.08% |
| 4 | 3.8197 | -111.014 | 4.2705 | -111.790 | 4.5492 | -111.272 | 5.0000 | -108.922 | 0.7295 | 4.2705 | 10.56% |
| 5 | 3.8197 | -111.014 | 4.0983 | -111.735 | 4.2705 | -111.790 | 4.5492 | -111.272 | 0.4508 | 4.2705 | 6.52% |
| 6 | 4.0983 | -111.735 | 4.2705 | -111.790 | 4.3769 | -111.680 | 4.5492 | -111.272 | 0.2786 | 4.2705 | 4.03% |
| 7 | 4.0983 | -111.735 | 4.2047 | -111.804 | 4.2705 | -111.790 | 4.3769 | -111.680 | 0.1722 | 4.2047 | 2.53% |
| 8 | 4.0983 | -111.735 | 4.1641 | -111.791 | 4.2047 | -111.804 | 4.2705 | -111.790 | 0.1064 | 4.2047 | 1.56% |
| 9 | 4.1641 | -111.791 | 4.2047 | -111.804 | 4.2299 | -111.804 | 4.2705 | -111.790 | 0.0658 | 4.2047 | 0.97% |
| 10 | 4.1641 | -111.791 | 4.1892 | -111.801 | 4.2047 | -111.804 | 4.2299 | -111.804 | 0.0407 | 4.2047 | 0.60% |

After ten iterations, the process falls below a stopping of 1% criterion and the result is converging on the true minimum at x = 4.2167 where the function has a value of y = -111.805.

Although it is not asked in the problem statement, here is how the same result could be determined with MATLAB:

clear, clc

format long, format compact

v0=-60;g=9.81;c=15;m=90;

f=@(t) m*g/c*t+m/c*(v0-m*g/c)*(1-exp(-c/m*t));

tmax=12;t=[0:tmax/100:tmax];

z=f(t);

plot(t,z)

set(gca,'Ydir','reverse'), grid

[tmax,zmax]=fminbnd(f,0,70)

tmax =

4.216709218815586

zmax =

-1.118045952080293e+02

Solution continued on next page...