Solution 7.35

(a) The fminbnd function can be used to determine the solution as in the following script:

clear,clc,format short g

D=@(V) 0.01*(0.6)*V^2+0.95/0.6*(16000/V)^2;

[Vmin,Dmin]=fminbnd(D,0,1200)

Vmin =

509.82

Dmin =

3119

(b) The following script can be used to generate the sensitivity analysis:

clear,clc,format short g

W=[12000:1000:20000];

sigma=0.6;

for i=1:length(W)

[Vmin(i),Dmin(i)]=fminbnd(@(V) 0.01*sigma*V^2+0.95/sigma*(W(i)/V)^2,0,1200);

end

z=[W;Vmin;Dmin];

fprintf('     Weight   Velocity   Drag\n')

fprintf('%10.0f %10.3f %10.3f\n',z)

clf

plot(W,Vmin,'--',W,Dmin)

legend('Velocity','Drag','location','best')

The results indicate that the minimum drag is fairly linear for the specified range.

Weight   Velocity   Drag

12000    441.515   2339.231

13000    459.544   2534.167

14000    476.891   2729.102

15000    493.629   2924.038

16000    509.818   3118.974

17000    525.508   3313.910

18000    540.744   3508.846

19000    555.561   3703.782

20000    569.994   3898.718