Solution 7.41

The following script can be developed to determine the location and magnitude of the maximum deflection with MATLAB’s fminbnd function,

clear;clc;clf

w=4*1e3*100;L=400/100;E=52000*1e3*1e4;I=32000/100^4;

f = @(x) -w/(48*E*I)*(2*x.^4-3*L*x.^3+L^3*x);

xp=[0:L/16:L];

yp=f(xp);

plot(xp,yp),grid

xlabel('x (m)'),ylabel('y (m)')

title('Deflection versus distance')

options=optimset('display','iter');

format long

[xopt,opt]=fminbnd(f,0,L,options);

xopt

maxdeflection=-opt

When this is run, the result is

Func-count     x          f(x)         Procedure

1        1.52786  -0.00329941        initial

2        2.47214  -0.00258495        golden

3        0.944272 -0.00260016        golden

4        1.70431  -0.00333256        parabolic

5        1.68674    -0.003333        parabolic

6        1.68621    -0.003333        parabolic

7        1.68614    -0.003333        parabolic

8        1.68617    -0.003333        parabolic

9        1.68611    -0.003333        parabolic

Optimization terminated:

the current x satisfies the termination criteria using OPTIONS.TolX of 1.000000e-04

xopt =

1.686140310219162

maxdeflection =

0.003332997911279