Problem 8.16

Besides solving simultaneous equations, linear algebra has lots of other applications in engineering and science. An example from computer graphics involves rotating an object in Euclidean space. The following rotation matrix can be employed to rotate a group of points counter-clockwise through an angle θ about the origin of a Cartesian coordinate system,

To do this, each point’s position must be represented by a column vector υ, containing the coordinates of the point. For example, here are vectors for the x and y coordinates of the rectangle in Fig. P8.16

x = [1 4 4 1]; y = [1 1 4 4];

The rotated vector is then generated with matrix multiplication: [R]{υ}. Develop a MATLAB function to perform this operation and display the initial and the rotated points as filled shapes on the same graph.

Problem continued on next page...

Here is a script to test your function:

clc;clf;format compact

x = [1 4 4 1]; y = [1 1 4 4];

[xt, yt] = Rotate2D(45, x, y);

and here is a skeleton of the function

function [xr, yr] = Rotate2D(thetad, x, y)

% two dimensional rotation 2D rotate Cartesian

% [xr, yr] = rot2d(thetad, x, y)

% Rotation of a two-dimensional object the Cartesian coordinates

% of which are contained in the vectors x and y.

% input:

% thetad = angle of rotation (degrees)

% x = vector containing objects x coordinates

% y = vector containing objects y coordinates

% output:

% xr = vector containing objects rotated x coordinates

% yr = vector containing objects rotated y coordinates

% convert angle to radians and set up rotation matrix

∙

∙ ·

∙

% close shape

∙

∙ ·

∙

% plot original object

hold on, grid on

∙

∙ ·

∙

% rotate shape

∙

∙ ·

∙

% plot rotated object

∙

∙ ·

∙

hold off