Problem 9.12

As described in Sec. 9.4, linear algebraic equations can arise in the solution of differential equations. For example, the following differential equation results from a steady-state mass balance for a chemical in a one-dimensional canal:

where c = concentration, t = time, x = distance, D = diffusion coefficient, U = fluid velocity, and k = a first-order decay rate. Convert this differential equation to an equivalent system of simultaneous algebraic equations. Given D = 2, U = 1, k = 0.2, c(0) = 80 and c(10) = 20, solve these equations from x = 0 to 10 and develop a plot of concentration versus distance.