Problem 9.18

As described in Sec. 9.5, linear algebraic equations can arise in the solution of differential equations. The following differential equation results from a steady-state mass balance for a chemical in a one-dimensional canal,

where x = distance along the canal (m), c = concentration, t = time, x = distance, D = diffusion coefficient, U = fluid velocity, and k = a first-order decay rate.

(a) Convert this differential equation to an equivalent system of simultaneous algebraic equations using centered difference approximations for the derivatives.

(b) Develop a function to solve these equations from x = 0 to L and return the resulting distances and concentrations. The first line of your function should be

function [x,c] = YourLastName_reactor(D, U, k, c0, cL, L, dx)

(c) Develop a script that invokes this function and then plots the results.

(d) Test your script for the following parameters: L = 10 m, Δx = 0.5 m, D = 2 m2/d, U = 1 m/d, k = 0.2/d, c(0) = 80 mg/L, and c(10) = 20 mg/L.