Problem 9.19

The following differential equation results from a force balance for a beam with a uniform loading,

where x = distance along the beam (m), y = deflection (m), L = length (m), E = modulus of elasticity (N/m2), I = moment of inertia (m4), and w = uniform load (N/m).

(a) Convert this differential equation to an equivalent system of simultaneous algebraic equations using a centered-difference approximation for the second derivative.

(b) Develop a function to solve these equations from x = 0 to L and return the resulting distances and deflections. The first line of your function should be

function [x, y] = YourLastName_beam (E, I, w, y0, yL, L, dx)

(c) Develop a script that invokes this function and then plots the results.

(d) Test your script for the following parameters: L = 3 m, Δx = 0.2 m, E = 250 × 109 N/m2,
 I = 3 × 10− 4 m4, w = 22,500 N/m, y(0) = 0, and y(3) = 0.