Problem 9.20

Heat is conducted along a metal rod positioned between two fixed temperature walls. Aside from conduction, heat is transferred between the rod and the surrounding air by convection. Based on a heat balance, the distribution of temperature along the rod is described by the following second-order differential equation

where T = temperature (K), h′ = a bulk heat transfer coefficient reflecting the relative importance of convection to conduction (m−2), x = distance along the rod (m), and T∞ = temperature of the surrounding fluid (K).

(a) Convert this differential equation to an equivalent system of simultaneous algebraic equations using a centered difference approximation for the second derivative.

(b) Develop a function to solve these equations from x = 0 to L and return the resulting distances and temperatures. The first line of your function should be

function [x, y] = YourLastName_rod(hp, Tinf, T0, TL, L, dx)

(c) Develop a script that invokes this function and then plots the results.

(d) Test your script for the following parameters: h′ = 0.0425 m−2, L = 12 m, T∞ = 220 K,
 T(0) = 320 K, T(L) = 450 K, and Δx = 0.5 m.