Problem 11.5

Determine the matrix inverse for the system described in Prob. 8.10. Use the matrix inverse to determine the force in the three members (F1, F2, and F3) if the vertical load at node 1 is doubled to F1,υ = −2000 N and a horizontal load of F3,h = −500 N is applied to node 3.

Problem 8.10

An important problem in structural engineering is that of finding the forces in a statically determinate truss (Fig. P8.10). This type of structure can be described as a system of coupled linear algebraic equations derived from force balances. The sum of the forces in both horizontal and

vertical directions must be zero at each node, because the system is at rest. Therefore,

for node 1:

ΣFH = 0 = −F1 cos 30° + F3 cos 60° + F1,h

ΣFV = 0 = −F1 sin 30° − F3 sin 60° + F1,υ

for node 2:

ΣFH = 0 = F2 + F1 cos 30° + F2,h + H2

ΣFV = 0 = F1 sin 30° + F2, υ + V2

for node 3:

ΣFH = 0 = −F2 − F3 cos 60° + F3,h

ΣFV = 0 = F3 sin 60° + F3, υ + V3

where Fi,h is the external horizontal force applied to node i (where a positive force is from left to right) and Fi,υ is the external vertical force applied to node i (where a positive force is upward).

Problem continued on next page

Thus, in this problem, the 2000-N downward force on node 1 corresponds to Fi,υ = −2000. For this case, all other Fi,υ’s and Fi,h’s are zero. Express this set of linear algebraic equations in matrix form and then use MATLAB to solve for the unknowns.