Problem 11.20

Figure P11.20 shows a statically determinate truss.

This type of structure can be described as a system of coupled linear algebraic equations by developing free-body force diagrams for the forces at each node in Fig. P11.20. The sum of the forces in both horizontal and vertical directions must be zero at each node, because the system is at rest. Therefore, for node 1,

FH = 0 = −F1 cos 30° + F3 cos 60° + F1,h

FV = 0 = −F1 sin 30° − F3 sin 60° + F1,υ

for node 2,

FH = 0 = F2 + F1 cos 30° + F2,h + H2

FV = 0 = F1 sin 30° + F2,υ + V2

for node 3,

FH = 0 = − F2 − F3 cos 60° + F3,h

FV = 0 = F3 sin 60° + F3,υ + V3

Problem continued on next page...

where Fi,h is the external horizontal force applied to node i (where a positive force is from left to right) and F1,υ is the external vertical force applied to node i (where a positive force is upward).

Thus, in this problem, the 1000-N downward force on node 1 corresponds to F1,υ =−1000. For this case all other Fi,υ ’s and Fi,h’s are zero. Note that the directions of the internal forces and reactions are unknown. Proper application of Newton’s laws requires only consistent assumptions regarding direction. Solutions are negative if the directions are assumed incorrectly. Also note that in this problem, the forces in all members are assumed to be in tension and act to pull adjoining nodes together. A negative solution therefore corresponds to compression. When the external forces are substituted and the trigonometric functions evaluated, this problem reduces to a set of six linear algebraic equations with six unknowns.

(a) Solve for the forces and reactions for the case displayed in Fig. P11.20.

(b) Determine the system’s matrix inverse. What is your interpretation of the zeros in the second row of the inverse?

(c) Use the elements matrix inverse to answer the following questions:

(i) If the force at node 1 was reversed (i.e., directed upward), compute the impact on H2 and V2.

(ii) If the force at node 1 was set to zero and horizontal forces of 1500 N were applied at nodes 1 and 2 (Fi,h = F2,h = 1500), what would be the vertical reaction at node 3 (V3).