Solution 11.11

First, the Vandermonde matrix can be set up

x1 = 4;x2=2;x3=7;x4=10;x5=3;x6=5;

A = [x1^5 x1^4 x1^3 x1^2 x1 1;x2^5 x2^4 x2^3 x2^2 x2 1;x3^5 x3^4 x3^3 x3^2 x3 1;x4^5 x4^4 x4^3 x4^2 x4 1;x5^5 x5^4 x5^3 x5^2 x5 1;x6^5 x6^4 x6^3 x6^2 x6 1]

A =

1024         256          64          16           4           1

32          16           8           4           2           1

16807        2401         343          49           7           1

100000       10000        1000         100          10           1

243          81          27           9           3           1

3125         625         125          25           5           1

Thus, about 7 digits might be suspect. A right-hand side vector can be developed corresponding to a solution of ones:

b=[sum(A(1,:));sum(A(2,:));sum(A(3,:));sum(A(4,:));sum(A(5,:)); sum(A(6,:))]

b =

1365

63

19608

111111

364

3906

The spectral condition number can be evaluated as

N = cond(A)

N =

1.4492e+007

The digits of precision that could be lost due to ill-conditioning can be calculated as

c = log10(N)

c =

7.1611

The solution can then be generated by left division

format long

x=A\b

x =

1.000000000000009

0.999999999999778

1.000000000002020

0.999999999991468

1.000000000016823

0.999999999987599

Solution continued on next page...

The maximum and mean errors can be computed as

emax = max(abs(x-1))

emean = mean(abs(x-1))

emax =

1.682254335833022e-11

emean =

6.667814448727920e-12

Some of the results are accurate to about 12 significant digits. Because MATLAB represents numbers to about 15 significant digits, this means that about 3 digits are suspect. Thus, for this case, the condition number tends to exaggerate the impact of ill-conditioning.