Solution 11.12

(a) The solution can be developed using MATLAB,

>> A=[13.422 0 0 0;

-13.422 12.252 0 0;

0 -12.252 12.377 0;

0 0 -12.377 11.797];

>> W=[750.5 300 102 30]';

>> AI=inv(A)

AI =

0.0745         0         0         0

0.0816    0.0816         0         0

0.0808    0.0808    0.0808         0

0.0848    0.0848    0.0848    0.0848

>> C=AI*W

C =

55.9157

85.7411

93.1163

100.2373

(b) The element of the matrix that relates the concentration of Havasu (lake 4) to the loading of Powell (lake 1) is  This value can be used to compute how much the loading to Lake Powell must be reduced in order for the chloride concentration of Lake Havasu to be 75 as

(c) First, normalize the matrix to give

Solution continued on next page...

The column-sum norm for this matrix is 2. The inverse of the matrix can be computed as

The column-sum norm for the inverse can be computed as 4.317672. The condition number is, therefore, 2(4.317672) = 8.635345. This means that less than 1 digit is suspect [log10(8.635345) = 0.93628]. Interestingly, if the original matrix is unscaled, the same condition number results.