Solution 11.20

The force balances can be expressed in matrix form as

(a) A Matlab script can be developed to solve for the forces and reactions for the case displayed in Fig. P11.20.

clear,clc

format short g

A=zeros(6); r_per_d=pi/180;

A(1,1)=cos(30*r_per_d); A(1,3)=-cos(60*r_per_d);

A(2,1)=sin(30*r_per_d); A(2,3)=sin(60*r_per_d);

A(3,1)=-cos(30*r_per_d); A(3,2)=-1; A(3,4)=-1;

A(4,1)=-sin(30*r_per_d); A(4,5)=-1;

A(5,2)=1; A(5,3)=cos(60*r_per_d);

A(6,3)=-sin(60*r_per_d); A(6,6)=-1;

b=[0 -1000 0 0 0 0]';

F=A\b

F =

-500

433.01

-866.03

0

250

750

(b) Determine the system’s matrix inverse.

Ainv=inv(A)

Ainv =

0.86603          0.5            0            0            0            0

0.25     -0.43301            0            0            1            0

-0.5      0.86603            0            0            0            0

-1            0           -1            0

-1            0

-0.43301        -0.25            0           -1            0            0

0.43301        -0.75            0            0            0           -1

Solution continued on next page...

The zeros in the second row mean that the force in the second member is not impacted by the forces applied to node 2 and the vertical force at node 3. This makes sense because (a) the second member is horizontal so its reaction would not be influenced by the vertical forces applied at its ends (nodes 2 and 3), and (b) a horizontal force applied at node 2 would have no effect because of the roller at node 3.

(c) (i) The key coefficients of the inverse that let us answer the question are 0 and –0.25, which give us the impacts of the node 1 vertical load on the horizontal and vertical reactions at node 2. Therefore

H2 = 1000(0) = 0

V2 = 1000(–0.25) = –250

(ii) The key coefficients of the inverse that let us answer the question are 0.43301 and 0, which give us the impacts of the horizontal loads at node 1 and node 2 on the vertical reactions at node 3. Therefore,

V3 = 0.43301(1500) + 0(1500) = 649.515