Problem 12.18

The following chemical reactions take place in a closed system

2A + B ⇌ C

A + D ⇌ C

At equilibrium, they can be characterized by

where the nomenclature ci represents the concentration of constituent i. If x1 and x2 are the number of moles of C that are produced due to the first and second reactions, respectively, use an approach to reformulate the equilibrium relationships in terms of the initial concentrations of the constituents. Then, solve the pair of simultaneous nonlinear equations for x1 and x2 if

K1 = 4 × 10−4, K2 = 3.7 × 10−2, ca,0 = 50, cb,0 = 20, cc,0 = 5, and cd,0 = 10.

(a) Use a graphical approach to develop your initial guesses. Then use these guesses as the starting point to determine refined estimates with

(b) the Newton-Raphson method, and

(c) the fsolve function.