Solution 12.11

Substituting centered difference finite differences, the Laplace equation can be written for the node (1, 1) as

Because the grid is square (x = y), this equation can be expressed as

The boundary node values (T01 = 100 and T10 = 75) can be substituted to give

The same approach can be written for the other interior nodes. When this is done, the following system of equations results

These equations can be solved using the Gauss–Seidel method. For example, the first iteration would be

Solution continued on next page...

The computation can be continued as follows:

| iteration | unknown | value | a | maximum a |
| --- | --- | --- | --- | --- |
| 1 | x1 | 43.75 | 100.00% |  |
|  | x2 | 42.1875 | 100.00% |  |
|  | x3 | 29.6875 | 100.00% |  |
|  | x4 | 24.21875 | 100.00% | 100.00% |
| 2 | x1 | 61.71875 | 29.11% |  |
|  | x2 | 52.73438 | 20.00% |  |
|  | x3 | 40.23438 | 26.21% |  |
|  | x4 | 29.49219 | 17.88% | 29.11% |
| 3 | x1 | 66.99219 | 7.87% |  |
|  | x2 | 55.37109 | 4.76% |  |
|  | x3 | 42.87109 | 6.15% |  |
|  | x4 | 30.81055 | 4.28% | 7.87% |
| 4 | x1 | 68.31055 | 1.93% |  |
|  | x2 | 56.03027 | 1.18% |  |
|  | x3 | 43.53027 | 1.51% |  |
|  | x4 | 31.14014 | 1.06% | 1.93% |
| 5 | x1 | 68.64014 | 0.48% |  |
|  | x2 | 56.19507 | 0.29% |  |
|  | x3 | 43.69507 | 0.38% |  |
|  | x4 | 31.22253 | 0.26% | 0.48% |

Thus, after 5 iterations, the maximum error is 0.48% and we are converging on the final result: T11 = 68.75, T12 = 56.25, T21 = 43.75, and T22 = 31.25.