Solution 12.19

The function along with the suggested script are:

clc, format compact

xguess=[7;7;3;7;3];

options=optimset('tolfun',1e-12)

[x1,fx1] = fsolve(@funpH,xguess,options,315);

[x2,fx2] = fsolve(@funpH,xguess,options,400);

x1',fx1',x2',fx2'

function f = funpH(x,pCO2)

% 10^(-x(1))=H, 10^(-x(2))=OH, 10^(-x(3))=HCO3, 10^(-x(4))=CO3, 10^(-x(5))=cT

KH=10^(-1.46); K1=10^(-6.3);K2=10^(-10.3);Kw=10^(-14);

f = [10^6*10^(-x(1))*10^(-x(3))/(KH*pCO2)-K1;...

10^(-x(1))*10^(-x(4))/10^(-x(3))-K2;...

10^(-x(1))*10^(-x(2))-Kw;...

(KH*pCO2)/1e6+10^(-x(3))+10^(-x(4))-10^(-x(5));...

10^(-x(3))+2*10^(-x(4))+10^(-x(2))-10^(-x(1))];

Results:

Equation solved.

fsolve completed because the vector of function values is near zero

as measured by the selected value of the function tolerance, and

the problem appears regular as measured by the gradient.

<stopping criteria details>

Equation solved.

fsolve completed because the vector of function values is near zero

as measured by the selected value of the function tolerance, and

the problem appears regular as measured by the gradient.

<stopping criteria details>

ans =

5.6304    8.3696    5.6312   10.3008    4.8774

ans =

1.0e-08 *

0.0000   -0.0000   -0.0000   -0.1069    0.0038

ans =

5.5787    8.4213    5.5793   10.3006    4.7824

ans =

1.0e-09 *

0.0000   -0.0000   -0.0000   -0.7908    0.0157