Problem 13.1

Repeat Example 13.1 but for three masses with the m’s = 40 kg and the k’s = 240 N/m. Produce a plot like Fig. 13.4 to identify the principle modes of vibration.

Problem Statement of Example 13.1

Use the polynomial method to solve for the eigenvalues of the following homogeneous system:

(10 − λ)x1 − 5x2 = 0

−5x1 + (10 − λ)x2 = 0