Problem 13.16

Recall that at the start of Chap. 8, we suspended three bungee jumpers with zero drag from frictionless cords that followed Hooke’s law. Determine the resulting eigenvalues and eigenvectors that would eventually characterize the jumpers’ oscillating motions and relative positions if they were instantaneously released from their starting positions as depicted in Fig. 8.1a (i.e., with each cord fully extended, but not stretched). Although bungee cords do not actually behave like true springs, assume that they stretch and compress in linear proportion to the applied force. Use the parameters from Example 8.2 (see below).