Solution 13.4

By summing forces on each mass and equating that to the mass times acceleration, the resu1ting differential equations can be written

In matrix form

The k/m matrix becomes with: k1 = k4 = 15 N/m, k2 = k3 = 35 N/m, and m1 = m2 = m3 = 1.5 kg

Solution continued on next page...

Solve for the eigenvalues/natural frequencies using MATLAB:

>> k1=15;k4=15;k2=35;k3=35;

>> m1=1.5;m2=1.5;m3=1.5;

>> a=[(k1+k2)/m1 -k2/m1 0;-k2/m2 (k2+k3)/m2 -k3/m2;0 -k3/m3 (k3+k4)/m3]

a =

33.3333  -23.3333         0

-23.3333   46.6667  -23.3333

0  -23.3333   33.3333

>> w2=eig(a)

w2 =

6.3350

33.3333

73.6650

>> w=sqrt(w2)

w =

2.5169

5.7735

8.5828