Solution 13.8

Removing the third floor reduces the system to:

Substituting the parameter values gives

Script can be written as

clear,clc,format compact

A=[450 -200;-240 240];

[v,d]=eig(A)

wn=sqrt(diag(d))'/2/pi

which yields

v =

0.8232    0.4983

-0.5678    0.8670

d =

587.9506         0

0  102.0494

wn =

3.8591    1.6078

Thus, the frequencies of the two modes are 3.8591 and 1.6078 Hz with the former oscillating in opposite directions and the latter together. We can develop a plot like Fig. 13.6 to visualize the modes: