Problem 14.36

Stokes Settling Law provides a means to compute the settling velocity of spherical particles under laminar conditions

where υs = the terminal settling velocity (m/s), g = gravitational acceleration (= 9.81 m/s2), ρ = the fluid density (kg/m3), ρs = the particle density (kg/m3), μ = the dynamic viscosity of the fluid (N s/m2), and d = the particle diameter (m). Suppose that you conduct an experiment in which you measure the terminal settling velocities of a number of 10-μm spheres having different densities,

| ρs, kg/m3 | 1500 | 1600 | 1700 | 1800 | 1900 | 2000 | 2100 | 2200 | 2300 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| υs, 10−3m/s | 1.03 | 1.12 | 1.59 | 1.76 | 2.42 | 2.51 | 3.06 | 3 | 3.5 |

(a) Generate a labeled plot of the data.

(b) Fit a straight line to the data with linear regression (polyfit) and superimpose this line on your plot.

(c) Use the model to predict the settling velocity of a 2500 kg/m3 density sphere.

(d) Use the slope and the intercept to estimate the fluid’s viscosity and density.