Problem 14.37

Beyond the examples in Fig. 14.13, there are other models that can be linearized using transformations. For example, the following model applies to third-order chemical reactions in batch reactors

where c = concentration (mg/L), c0 = initial concentration (mg/L), k = reaction rate 
(L2/(mg2 d)), and t = time (d). Linearize this model and use it to estimate k and c0 based on the following data. Develop plots of your fit along with the data for both the transformed, linearized form and the untransformed form.

| t | 0 | 0.5 | 1 | 1.5 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| c | 3.26 | 2.09 | 1.62 | 1.48 | 1.17 | 1.06 | 0.9 | 0.85 |