Problem 14.41

Repeat Prob. 14.40, but for a two-dimensional random walk. As depicted in Fig. P14.41, have each particle take a random step of length Δ at a random angle θ ranging from 0 to 2π. Generate an animated two panel stacked plot with the location of all the particles displayed on the top plot (subplot (2,1,1)), and the histogram of the particles’ x coordinates on the bottom (subplot (2,1,2)).

Problem 14.40

Suppose that a population of particles is confined to motion along a one-dimensional line (Fig. P14.40). Assume that each particle has an equal likelihood of moving a distance, Δx, to either the left or right over a time step, Δt. At t = 0 all particles are grouped at x = 0 and are allowed to take one step in either direction. After Δt, approximately 50% will step to the right and 50% to the left. After 2Δt, 25% would be two steps to the left, 25% would be two steps to the right, and 50% would have stepped back to the origin. With additional time, the particles would spread out with the population greater near the origin and diminishing at the ends. The net result is that the distribution of the particles approaches a spreading bell-shaped distribution. This process, formally called a random walk (or drunkard’s walk), describes many phenomena in engineering and science with a common example being Brownian motion. Develop a MATLAB function that is given a stepsize (Δx), and a total number of particles (n) and steps (m).

Problem continued on next page...

At each step, determine the location along the x axis of each particle and use these results to generate an animated histogram that displays how the distribution’s shape evolves as the computation progresses.