Solution 14.9

(a) The data can be plotted

The plot indicates that the data is somewhat curvilinear. An exponential model (i.e., a semi-log plot) is the best choice to linearize the data. This conclusion is based on

- A power model does not result in a linear plot

- Bacterial decay is known to follow an exponential model

- The exponential model by definition will not produce negative values.

The exponential fit can be determined as

| t (hrs) | c (CFU/100 mL) | ln c |
| --- | --- | --- |
| 4 | 1600 | 7.377759 |
| 8 | 1320 | 7.185387 |
| 12 | 1000 | 6.907755 |
| 16 | 890 | 6.791221 |
| 20 | 650 | 6.476972 |
| 24 | 560 | 6.327937 |

Solution continued on next page...

Therefore, the coefficient of the exponent (1) is –0.0535 and the lead coefficient (1) is e7.5936 = 1985.437, and the fit is

Consequently the concentration at t = 0 is 1985.437 CFU/100 ml. Here is a plot of the fit along with the original data:

(b) The time at which the concentration will reach 200 CFU/100 mL can be computed as