Solution 14.16

function [a, r2, syx] = linregr2(x,y)

% [a, r2, syx] = linregr(x,y):

%   Least squares fit of a straight line to data

%   by solving the normal equations.

% input:

%   x = independent variable

%   y = dependent variable

% output:

%   a = vector of slope, a(1), and intercept, a(2)

%   r2 = coefficient of determination

%   syx = standard error of the estimate

n = length(x);

if length(y)~=n, error('x and y must be same length'); end

x = x(:); y = y(:);    % convert to column vectors

sx = sum(x); sy = sum(y);

sx2 = sum(x.*x); sxy = sum(x.*y); sy2 = sum(y.*y);

a(1) = (n*sxy-sx*sy)/(n*sx2-sx^2);

a(2) = sy/n-a(1)*sx/n;

syx=sqrt(sum((y-a(1)*x-a(2)).^2)/(n-2));

r2 = ((n*sxy-sx*sy)/sqrt(n*sx2-sx^2)/sqrt(n*sy2-sy^2))^2;

% create plot of data and best fit line

xp = linspace(min(x),max(x),2);

yp = a(1)*xp+a(2);

res=a(1)*x+a(2)-y;

subplot(2,1,1);plot(x,y,'o',xp,yp)

xlabel('x'),ylabel('y'),title('Linear Least-Squares Fit')

subplot(2,1,2);plot(x,res,'o',x,res)

xlabel('x'),ylabel('residual'),title('Residual Plot')

grid on

Test application to data from Table 14.1.

>> x=[10 20 30 40 50 60 70 80];

>> y=[25 70 380 550 610 1220 830 1450];

>> [a, r2, syx] = linregr2(x,y)

a =

19.4702 -234.2857

r2 =

0.8805

syx =

189.7885