Solution 14.17

function [b, r2] = powerfit(x,y)

% linregr: linear regression curve fitting

%   [b, r2] = powerfit(x,y): Least squares fit of straight

%            line to log-transformed data

% input:

%   x = independent variable

%   y = dependent variable

% output:

%   b = vector of coefficient, b(1), and exponent, b(2)

%   r2 = coefficient of determination

n = length(x);

if length(y)~=n, error('x and y must be same length'); end

x = x(:); y = y(:);

xl=log10(x);yl=log10(y);

sx = sum(xl); sy = sum(yl);

sx2 = sum(xl.*xl); sxy = sum(xl.*yl); sy2 = sum(yl.*yl);

% compute slope and intercept

a(1) = (n*sxy-sx*sy)/(n*sx2-sx^2);

a(2) = sy/n-a(1)*sx/n;

% compute coefficient and exponent

b(1) = 10^a(2);

b(2) = a(1);

St=sum((mean(y)-y).^2);

ypred = b(1)*x.^b(2);

Sr=sum((ypred-y).^2);

r2 = (St - Sr)/St;

% create plots of data and best fit line

xp = linspace(min(x),max(x));

yp = b(1)*xp.^b(2);

subplot(2,1,1);plot(x,y,'o',xp,yp)

plot(x,y,'o',xp,yp)

xlabel('x'),ylabel('y'),title('Fit (untransformed)')

subplot(2,1,2);loglog(x,y,'o',xp,yp)

xlabel('log(x)'),ylabel('log(y)'),title('Fit (transformed)')

grid on

Solution continued on next page...

Script to solve Prob. 14.11:

clear,clc,clf

mass=[400 70 45 2 0.3 0.16];

metabolism=[270 82 50 4.8 1.45 0.97];

[b,r2]=powerfit(mass,metabolism)

b =

3.3893    0.7266

r2 =

0.9978