Solution 14.26

First, we can determine the stress

We can then try to fit the data to obtain a mathematical relationship between strain and stress. First, we can try linear regression:

clear,clc,clf,format short g,format compact

strain=[0.0032 0.0045 0.0055 0.0016 0.0085 0.0005];

stress=[4970 5170 5500 3590 6900 1240];

a = polyfit(stress,strain,1)

stressp=linspace(min(stress),max(stress));

strainp = polyval(a, stressp);

plot(stressp,strainp,stress,strain,'o'), grid

a =

1.3712e-06   -0.0022885

We can use this model to compute the strain and deflection

= 1.3712410–6(2347.4178) – 2.2884910–3 = 9.303810–4

L = 9.303810–4(9) = 0.0083735 m

Solution continued on next page...

This is not a particularly good fit. In particular, the negative intercept is physically unrealistic. We therefore try a best-fit parabola:

a = polyfit(stress,strain,2)

strainp = polyval(a, stressp);

plot(stressp,strainp,stress,strain,'o'), grid

a =

2.8177e-10  -8.4598e-07    0.0010766

We can use this model to compute the strain and deflection

= 2.817710–10(2347.4178)2 – 8.459810–7(2347.4178) + 1.076610–3 = 6.434110–4

L = 6.434110–4 (9) = 0.0057907 m