Problem 15.7

As compared with the models from Probs. 15.5 and 15.6, a somewhat more sophisticated model that accounts for the effect of both temperature and chloride on dissolved oxygen saturation can be hypothesized as being of the form

o = f 3(T ) + f 1(c)

That is, a third-order polynomial in temperature and a linear relationship in chloride is assumed to yield superior results. Use the general linear least-squares approach to fit this model to the data in Table P15.5. Use the resulting equation to estimate the dissolved oxygen concentration for a chloride concentration of 15 g/L at T = 12 °C. Note that the true value is 9.09 mg/L. Compute the percent relative error for your prediction.

Table P15.5

Dissolved oxygen concentration in water as a function of temperature (°C) and chloride concentration (g/L).

Dissolved Oxygen (mg/L) for Temperature (°C)

and Concentration of Chloride (g/L)

| T, °C | C = 0 g/L | C = 10 g/L | C = 20 g/L |
| --- | --- | --- | --- |
| 0 | 14.6 | 12.9 | 11.4 |
| 5 | 12.8 | 11.3 | 10.3 |
| 10 | 11.3 | 10.1 | 8.96 |
| 15 | 10.1 | 9.03 | 8.08 |
| 20 | 9.09 | 8.17 | 7.35 |
| 25 | 8.26 | 7.46 | 6.73 |
| 30 | 7.56 | 6.85 | 6.20 |

Problem continued on next page...

Problem 15.5

For the data from Table P15.5, use polynomial regression to derive a predictive equation for dissolved oxygen concentration as a function of temperature for the case where the chloride concentration is equal to zero. Employ a polynomial that is of sufficiently high order that the predictions match the number of significant digits displayed in the table.

Problem 15.6

Use multiple linear regression to derive a predictive equation for dissolved oxygen concentration as a function of temperature and chloride based on the data from Table P15.5. Use the equation to estimate the concentration of dissolved oxygen for a chloride concentration of 15 g/L at T = 12 °C. Note that the true value is 9.09 mg/L. Compute the percent relative error for your prediction. Explain possible causes for the discrepancy.