Problem 15.29

The Antoine equation describes the relation between vapor pressure and temperature for pure components as

where p is the vapor pressure, T is temperature (K), and A, B, and C are component-specific constants. Use MATLAB to determine the best values for the constants for carbon monoxide based on the following measurements

| T (K) | 50 | 60 | 70 | 80 | 90 | 100 | 110 | 120 | 130 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| p (Pa) | 82 | 2300 | 18,500 | 80,500 | 2.3×105 | 5×105 | 9.6×105 | 1.5×106 | 2.4×106 |

In addition to the constants determine the r2 and sy∕x for your fit.